; 3.1

; a program to display a message by using MACRO

display macro msg				; display - user define macro

mov ah, 09h				; display string function

mov dx, offset msg           	; initialize dx to msg

int 21h				; call dos sevice

endm

clrscr macro                       	; to clear the screen

mov ah, 00h

mov al, 02h

int 10h

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data – name of user defined segment

cr     equ	0dh

lf     equ	0ah

msg1	db cr, lf,	'What do you want to display? - $'

msg2 	db cr, lf,	'You wanted to display - $'

msg3	db cr, lf, lf,	'Now press a if you want to input again or x to exit - $'

msg4 	db cr, lf, 	'You pressed x so the program has exit. $'

msg5 	db cr, lf, 	'Ah! Wrong choice! Its out of range! $'

a 	db 100 dup(?)  		; maximum characters

endm

.code				          	; code – name of user defined segment

assume   ds: @data,   cs: @code	; initialize ds and cs to segments

start:

mov ax, data

mov ds, ax

display msg1

lea si, a

mov cl, 00

up:

mov ah, 01h

int 21h

cmp al, 0dh

je display

mov [si], al

inc si

inc cl

cmp cl, 100         		; maximum characters

jl up

display:

lea si, a

clrscr				; to clear the screen

display msg2

l1:						; level 1

cmp cl, 00

je again

mov dl, [si]

mov ah, 02h

int 21h

inc si

dec cl

jmp l1

again:

mov ax, @data

mov ds, ax

display msg3			; Now press a if you want to input again or x to exit

mov ah, 01h         	; get input from keyboard

int 21h

cmp al, 'a'         	; compare with ASCII value of s

je start            	; if equal jump to start function

cmp al, 'x'         	; compare with ASCII value of x

je exit             	; if equal jump to exit function

wrong:

display msg5			; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg4	     		; You pressed x so the program has exit.

exit

endm					; end of code segment

end start

; 3.2

; a program to display any ten decimal numbers

display macro msg       	; display - user defined macro

mov ah, 09h			; display string function

mov dx, offset msg	; initialize dx to msg

int 21h			; call DOS service

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data 				; data - name of user defined segment

cr	equ	0dh		; ASCII code for carriage return

lf	equ	0ah		; ASCII code for line feed

tab 	equ	09h		; ASCII code for tab

msg1 db cr, lf,   'Ten consecutive decimal numbers to display.....$'

msg2 db cr, lf,   'From which number You want to start? - $'

msg3 db cr, lf, lf,'Now press a if you want to type again or x to exit - $'

msg4 db cr, lf, 	'You pressed x so the program has exit. $'

msg5 db cr, lf, 	'Ah! Wrong choice! Its out of range! $'

newline	db cr, lf,	'$'

result 	db 5 dup(?)	; result table

deci 	db	?

endm   				; end of data segment

.code					; code - name of user defined segment

assume ds: @data, cs: @code   ; initialize ds and cs to segments

start:

mov ax, @data		; initialize ds to the data segment

mov ds, ax

display msg1

display msg2

mov ah, 01h			; get character from keyboard

int 21h			; call DOS service

sub al, '0'			; ASCII number to decimal

mov bx, 10			; load 10 in bx

mul bl			; multiply with ten

mov deci, al		; store first digit in num

mov ah, 01h			; get character from keyboard

int 21h			; call DOS service

sub al, '0'			; ASCII number to decimal

add deci, al		; add second digit

mov cx, 10			; initialize count register - ekahne koto porjonto count korbe ta nirdharon kora hobe

next_deci:				; loop for printing ten numbers

mov ah, 00h

mov al, deci

mov si, offset result

call hex2asc		; hexadecimal to ASCII conversion

display newline

display result		; print first number

inc deci

loop next_deci

again:

mov ax, @data

mov ds, ax

display msg3	; Now press a if you want to type again or x to exit -

mov ah, 01h       ; get input from keyboard

int 21h

cmp al, 'a'       ; compare with ASCII value of s

je start          ; if equal jump to start function

cmp al, 'x'       ; compare with ASCII value of x

je exit           ; if equal jump to exit function

wrong:

display msg5	; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg4	; You pressed x so the program has exit.

exit

hex2asc proc near		; function to convert hexa to ASCII

pusha			; save all registers

mov cx, 00h		; counter for intermediate data pushed

mov bx, 0ah		; load 10 in bl

rpt1:

mov dx, 00h

div bx		; divide deci by 10

add dl, '0'		; convert remainder to ASCII

push dx		; store ASCII digit on to the stack

inc cx		; update counter

cmp ax, 0ah		; is sum less than or equal to 10

jge rpt1		; if yes, perform conversion

add al, '0'		; convert last digit to ASCII

mov [si], al	; store last digit

rpt2:

pop ax			; pop data

inc si			; advance result string pointer

mov [si], al		; store in result

loop rpt2

inc si

mov al, '$'

mov [si], al		; append end of string

popa				; restore the registers

ret				; return to the main function

hex2asc endp			; end of procedure - endp/endm o likha jeto

endm					; end of segment

end start				; end of program

; 4.1

; a program to sum of n/4 different numbers

display macro msg

mov ah, 09h

mov dx, offset msg	; lea dx, msg

int 21h

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data

cr equ 0dh

lf equ 0ah

msg1 db cr, lf,'How many numbers do you want to sum? N = $'

msg2 db cr, lf,'Enter value:$'

msg3 db cr, lf,'Sum = $'

msg4 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg5 db cr, lf, 'You pressed x so the program has exit. $'

msg6 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

N db ?

num db ?

Buffer db 5 dup('$')

sum db 5 dup('$')

endm

.code

assume ds: @data, cs: @code

start:

mov ax, @data

mov ds,ax

display msg1

call readnum

mov al, N     ; N er jaygay aage thekei value dite pari

mov num, al

mov ch, 00h

mov cl, num

mov si, offset buffer

Rd_nums:

display msg2

call readnum

mov al, N

mov [si],al

inc si

loop Rd_nums

mov si, offset buffer

mov cl, num

mov al, [si]

mov ah, 00h

dec cl

le1:

inc si

mov bl, [si]

mov bh, 00h

add ax, bx

loop le1

mov si, offset sum

call hex2asc

display msg3

display sum

again:

mov ax, @data

mov ds, ax

display msg4        ; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of a

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg6		 ; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg5        ; You pressed x so the program has exit.

exit

readnum proc near

push ax

push bx

mov ah, 01h

int 21h

sub al, '0'

mov bh, 0ah

mul bh

mov N, al

mov ah, 01h

int 21h

cmp al, cr

je e1

sub al,'0'

add N, al

jmp e2

e1:

mov al, N

mov ah, 00h

div bh

mov N, al

e2:

pop bx

pop ax

ret

readnum endp

hex2asc proc near

push ax

push bx

push cx

push dx

push si

mov cx, 00h

mov bx, 0ah

rpt1:

mov dx, 00h

div bx

add dl, '0'

push dx

inc cx

cmp ax, 0ah

jge rpt1

add al, '0'

mov [si], al

rpt2:

pop ax

inc si

mov [si], al

loop rpt2

inc si

mov al, '$'

mov [si], al

pop si

pop dx

pop cx

pop bx

pop ax

ret

hex2asc endp

endm

end start

; 4.2

; a program to find odd or even numbers

display macro msg				; display - user defined macro

mov ah, 09h				; display string function

mov dx, offset msg		; initialize dx to msg

int 21h				; call DOS service

endm

readnum macro num				; macro to get a two digit input/number

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

mov bx, 10				; load 10 in bx

mul bl				; multiply with ten

mov num, al				; store first digit in num

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

add num, al				; add second digit

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data - name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

msg1 db cr, lf, 'Enter a two digit number: $'

msg2 db cr, lf, 'The Number You Entered Is Even. $'

msg3 db cr, lf, 'The Number You Entered Is Odd. $'

msg4 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg5 db cr, lf, 'You pressed x so the program has exit. $'

msg6 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

N    db	?

endm   					; end of data segment

.code						; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds, ax

display msg1

readnum N				; get number input

mov al, N

mov cl, 2				; load 2 in cl

div cl				; divide al by cl

cmp ah, 0				; compare remainder with zero

je  Even				; if equal jump to Even function

cmp ah, 1				; compare remainder with zero

je  Odd				; if equal jump to Odd function

Even:						; function to display even number

display msg2

jmp again

Odd:

display msg3

jmp again

again:

mov ax, @data

mov ds, ax

display msg4    	; Now press a if you want to type again or x to exit -

mov ah, 01h        ; get input from keyboard

int 21h

cmp al, 'a'        ; compare with ASCII value of s

je start           ; if equal jump to start function

cmp al, 'x'        ; compare with ASCII value of x

je exit            ; if equal jump to exit function

wrong:

display msg6		; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg5       ; You pressed x so the program has exit.

exit               ; exit program

endm				; end of segment

end start			; end of program

; 5.1

; a program to display any string in reverse order

display macro msg				; display - user defined macro

mov ah,09h				; display string function

mov dx,offset msg			; initialize dx to msg

int 21h				; call DOS service

endm

readchar macro char

mov ah,01h				; get a character from keyboard

int 21h				; call DOS service

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data - name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

msg1 db cr,lf,	'Enter your string: $'

msg2 db cr,lf,	'The reversed string is: $'

msg3 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg4 db cr, lf, 'You pressed x so the program has exit. $'

msg5 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

temp      db   ?

chartable db 100 dup (?)	; input character table

rev 	     db 100 dup ('$')	; reversed character table

endm 						; end of data segment

.code						; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds,ax

display msg1

mov bx,0

lea si,chartable			; load effective address of chartable

getchar:					; function to store input

readchar temp

mov ah,00h				; compare input character with

cmp al,0dh				; ASCII value of enter

je reverse				; if equal jump to reverse function

inc bl

mov [si+1],al			; store character in chartable

inc si				; advance input string pointer

loop getchar

reverse:

display msg2

mov cx,bx

rev_strng:					; function to reverse the string

mov al,[si]				; load the last character input in al

mov rev,al

display rev				; display the last character at first

dec si				; decrease output string pointer

loop rev_strng

again:

mov ax, @data

mov ds, ax

display msg3        ; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of s

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg5		; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg4       ; You pressed x so the program has exit.

exit               ; exit program

endm				; end of code segment

end start			; end of program

; 5.2

; a program to determine the length of a string

display macro msg				; display - user defined macro

mov ah, 09h				; display string function

mov dx, offset msg		; initialize dx to msg

int 21h				; call DOS service

endm

readchar macro char

mov ah,01h				; get a character from keyboard

int 21h				; call DOS service

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data – name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

msg1 db cr, lf, 'Enter Your String: $'

msg2 db cr, lf, 'String length     : $'

msg3 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg4 db cr, lf, 'You pressed x so the program has exit. $'

msg5 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

temp 		db 	?

stringlength 	db 	10 dup ('$')

endm   					; end of data segment

.code 					; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds, ax

display msg1

mov bx, 0

getchar:					; function to store input

readchar temp

cmp al, 0dh				; compare “input” with “enter”

je count

inc bl

loop getchar

count:					; function to determine length of string

mov cx, bx

display msg2

mov ax, bx

lea si, stringlength

call hex2asc			; hexadecimal to ASCII conversion call

display stringlength

again:

mov ax, @data

mov ds, ax

display msg3        ; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of s

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg5		 ; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg4	      	; You pressed x so the program has exit.

exit         	      ; exit program

hex2asc proc near			; function to convert hexadecimal to ASCII string

pusha					; save all registers

mov cx, 00h				; counter for intermediate data pushed

mov bx, 0ah				; load 10 in bl

rpt1:

mov dx, 00h

div bx				; divide deci by 10

add dl, '0'				; convert remainder to ASCII

push dx				; store ASCII digit on to the stack

inc cx				; update counter

cmp ax, 0ah				; is sum less than or equal to 10

jge rpt1				; if yes, perform conversion

add al, '0'				; convert last digit to ASCII

mov [si], al			; store last digit

rpt2:

pop ax					; pop data

inc si					; advance result string pointer

mov [si], al				; store in result

loop rpt2

inc si

mov al, '$'

mov [si], al				; append end of string

popa						; restore the registers

ret						; return to the main function

hex2asc endp				; end of procedure

endm						; end of segment

end start					; end of program

; 6.1

; a program to find the largest number from N numbers

display macro msg				; display - user defined macro

mov ah, 09h				; display string function

mov dx, offset msg		; initialize dx to msg

int 21h				; call DOS service

endm

readnum macro num				; macro to get a two digit input number

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

mov bx, 10				; load 10 in bx

mul bl				; multiply with ten

mov num, al				; store first digit in num

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

add num, al				; add second digit

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data - name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

msg1 db cr, lf, 'How many numbers do you want to compare? : $'

msg2 db cr, lf, 'Enter number: $'

msg3 db cr, lf, 'The largest number is: $'

msg4 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg5 db cr, lf, 'You pressed x so the program has exit. $'

msg6 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

N      db  ?

temp   db  ?

ntable db  100 dup (?)		; number table

result db  20  dup ('$')

endm   					; end of data segment

.code						; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds, ax

display msg1

readnum N				; How many numbers

mov si, offset ntable

mov ch, 0

mov cl, N

nextread:					; function to get input numbers

display msg2

readnum temp

mov al, temp

mov [si], al

inc si

loop nextread

lea si, ntable				; load effective address of ntable in pointer

mov bl, [si]

mov cl, 01

nextchk:						; function to find largest number

inc si

cmp cl, N

je nomore

cmp bl, [si]

jge skip     				; ekhane change kore largest or lowest select korte hobe: jge/jle

mov bl, [si]

skip:

inc cl

jmp nextchk

nomore:						; function to display largest number

mov ah, 0

mov al, bl

lea si, result

call hex2asc 				; Hexadecimal to ASCII conversion call

display msg3

display result

again:

mov ax, @data

mov ds, ax

display msg4        ; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of a

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg6		 ; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg5        ; You pressed x so the program has exit.

exit                ; exit program

hex2asc proc near				; function to convert hexadecimal to ASCII

pusha					; save all registers

mov cx, 00h				; counter for intermediate data pushed

mov bx, 0ah				; load 10 in bl

rpt1:

mov dx, 00h

div bx				; divide deci by 10

add dl, '0'				; convert remainder to ASCII

push dx				; store ASCII digit on to the stack

inc cx				; update counter

cmp ax, 0ah				; is sum less than or equal to 10

jge rpt1				; if yes, perform conversion

add al, '0'				; convert last digit to ASCII

mov [si], al			; store last digit

rpt2:

pop ax					; pop data

inc si					; advance result string pointer

mov [si], al				; store in result

loop rpt2

inc si

mov al, '$'

mov [si], al				; append end of string

popa						; restore the registers

ret						; return to the main function

hex2asc endp					; end of procedure

endm							; end of segment

end start						; end of program

; 6.2

; a program to sort 10 numbers in ascending order

display macro msg				; display - user defined macro

mov ah, 09h				; display string function

mov dx, offset msg		; initialize dx to msg

int 21h				; call DOS service

endm

readnum macro num				; macro to get a two digit input number

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

mov bx, 10				; load 10 in bx

mul bl				; multiply with ten

mov num, al				; store first digit in num

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

add num, al				; add second digit

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data 					; data - name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

tab	equ	09h			; ASCII code for tab

msg1 db cr, lf, 'Enter numbers: $'

msg2 db cr, lf, 'Numbers after sorting:', cr, lf, '$'

msg3 db tab, '$'

msg4 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg5 db cr, lf, 'You pressed x so the program has exit. $'

msg6 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

temp        db   ?

ntable      db   50 dup (?)

restable    db   50 dup ('$')

endm   					; end of data segment

.code segment				; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds, ax

mov cx, 10				; load 10 in count register - ekhane change kore number select korte hobe *

mov di, offset ntable		; load effective address of ntable in pointer

rdnxt:					; function to get the input numbers

display msg1

readnum temp

mov al, temp

mov [di], al

inc di

loop rdnxt

mov dh, 0

mov dl, 10

sort: 					; function to sort the numbers

dec dx

cmp dx, 0

je  print

mov cx, 10			; * for ascending

dec cx

mov di, offset ntable

lrgnum:					; function to compare two numbers

mov al, [di]

mov bl, [di+1]

cmp al, bl

jge skip 			; descending / ascending - jge/jle

mov [di], bl

mov [di+1], al

skip:

inc di

loop lrgnum

jmp sort

print:					; function to print the sorted numbers

display msg2

mov cx, 10			; * for descending

mov si, offset restable

mov di, offset ntable

loop1:

mov  ah, 0

mov  al, [di]

call hex2asc		; Hexadecimal to ASCII conversion

mov  ah, 09h

lea  dx, [si]

int  21h

inc  di

display msg3

loop loop1

again:

mov ax, @data

mov ds, ax

display msg4	; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of s

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg6		; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg5        ; You pressed x so the program has exit.

exit                ; exit program

hex2asc proc near				; function to convert hexadecimal to ASCII string

pusha					; save all registers

mov cx, 00h				; counter for intermediate data pushed

mov bx, 0ah				; load 10 in bl

rpt1:

mov dx, 00h

div bx				; divide deci by 10

add dl, '0'				; convert remainder to ASCII

push dx				; store ASCII digit on to the stack

inc cx				; update counter

cmp ax, 0ah				; is sum less than or equal to 10

jge rpt1				; if yes, perform conversion

add al, '0'				; convert last digit to ASCII

mov [si], al			; store last digit

rpt2:

pop ax					; pop data

inc si					; advance result string pointer

mov [si], al				; store in result

loop rpt2

inc si

mov al, '$'

mov [si], al				; append end of string

popa						; restore the registers

ret						; return to the main function

hex2asc endp					; end of procedure

endm							; end of segment

end start						; end of program

; 7

; a program to display any 10 decimal numbers and their corresponding ASCII characters

display macro msg				; display - user defined macro

mov ah, 09h				; display string function

mov dx, offset msg		; initialize dx to msg

int 21h				; call DOS service

endm

exit macro

mov ah, 4ch

mov al, 0h

int 21h

endm

.data						; data - name of user defined segment

cr	equ	0dh			; ASCII code for carriage return

lf	equ	0ah			; ASCII code for line feed

tab 	equ	09h			; ASCII code for tab

msg1 db cr, lf,'Ten different decimal numbers with ASCII to display......$'

msg2 db cr, lf,'From which number You want to start? $'

msg3 db cr, lf,'decimal',tab,'ASCII', '$'

msg4 db cr, lf, lf,	'Now press a if you want to type again or x to exit - $'

msg5 db cr, lf, 'You pressed x so the program has exit. $'

msg6 db cr, lf, 'Ah! Wrong choice! Its out of range! $'

tb   db tab, '$'

newline db cr,lf,'$'

result  db 5 dup (?)		; result table

deci    db 5 dup ('$')

endm   					; end of data segment

.code						; code - name of user defined segment

assume ds: @data, cs: @code		; initialize ds and cs to segments

start:

mov ax, @data			; initialize ds to  the data segment

mov ds, ax

display msg1

display msg2

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

mov bx, 10				; load 10 in bx

mul bl				; multiply with ten

mov deci, al			; store first digit in num

mov ah, 01h				; get character from keyboard

int 21h				; call DOS service

sub al, '0'				; ASCII number to decimal

add deci, al			; add second digit

mov cx, 10				; initialize count register - koto tuku count korte hobe

display msg3

next_deci:					; loop for printing ten numbers

mov ah, 00h

mov al, deci

mov si, offset result

call hex2asc			; hexadecimal to ASCII conversion

display newline

display result			; print first number

display tb

display deci			; display ASCII value

inc deci

loop next_deci

again:

mov ax, @data

mov ds, ax

display msg4        ; Now press a if you want to type again or x to exit -

mov ah, 01h         ; get input from keyboard

int 21h

cmp al, 'a'         ; compare with ASCII value of s

je start            ; if equal jump to start function

cmp al, 'x'         ; compare with ASCII value of x

je exit             ; if equal jump to exit function

wrong:

display msg6		 ; Ah! Wrong choice! Its out of range!

jmp again

exit:

display msg5        ; You pressed x so the program has exit

exit                ; exit program

hex2asc proc near				; function to convert hexadecimal to ASCII

pusha					; save all registers

mov cx, 00h				; counter for intermediate data pushed

mov bx, 0ah				; load 10 in bl

rpt1:

mov dx, 00h

div bx				; divide deci by 10

add dl, '0'				; convert remainder to ASCII

push dx				; store ASCII digit on to the stack

inc cx				; update counter

cmp ax, 0ah				; is sum less than or equal to 10

jge rpt1				; if yes, perform conversion

add al, '0'				; convert last digit to ASCII

mov [si], al			; store last digit

rpt2:

pop ax					; pop data

inc si					; advance result string pointer

mov [si], al				; store in result

loop rpt2

inc si

mov al, '$'

mov [si], al				; append end of string

popa						; restore the registers

ret						; return to the main function

hex2asc endp					; end of procedure

endm							; end of segment

end start						; end of program