MOV destination, source:

The function of the MOV instruction is to copy a data from source to destination.

Operands

Pneumonic

MOV AL, 8AH ; H indicates hexadecimal number

; the function of the above instruction is to copy 8AH into AL register. 8-bit data transfer

MOV AH, 7AH

; the function of the above instruction is to copy 7AH into AH register. 8-bit data transfer

MOV AX, 3B7AH; the function of the above instruction is to copy 3B7AH into AX register. 16-bit data transfer

AL = Multiplicand -> 8-bit

Multiplication:

X  Multiplier -> 8-bit

For 8086 microprocessor there are two types of multiplications.

- 8-bit multiplication

AX = Product     -> 16-bit

16-bit multiplication

8-bit multiplication:

The 8-bit multiplicand will be multiplied by 8-bit multiplicand and 16-bit product will be stored in AX register. Multiplicand must be in AL register, but the multiplier can be in any other multipurpose register.

CL = 10 = 0AH

AL = 25 = 19 H

AX = 250 = FA h

Example 1:

16-bit multiplication:

The 8-bit multiplicand will be multiplied by 8-bit multiplicand and 16-bit product will be stored in AX register. Multiplicand must be in AL register, but the multiplier can be in any other multipurpose register.

AX = Multiplicand -> 16-bit

DX-AX = Product     -> 32-bit

X  Multiplier -> 16-bit

AX = 32650 = 7F8A H

DX=218C, AX = F570H

Example 2:

CX = 17240 = 4358 H